TECHNICAL DATA SHEET

------------------------------- Segui il tuo Istinto-----------------------------

sez. 1.0 Technical characteristics

Segui il tuo Istinto is a wall covering made ​​from lime seasoned for at least 6 months, finely filtered. Easy to apply, it has excellent filling properties and it is naturally resistant to mold and bacteria.

Note: Advanced Spirito Libero recommends that our entire range of lime based natural materials is used only by highly skilled or licensed tradespeople with extensive experience in this field. It is the responsibility of the applicator to understand the application method and the procedure involved ,in order to obtain a satisfactory outcome. Advanced Spirito Libero recommends that architects, builders and clients appoint only experienced applicators in the relevant lime-based materials to ensure they'll be able to deliver the same finish as per the sample shown.

sez. 2.0 Use

Segui il tuo Istinto is suitable for decorating indoor wall surfaces.

sez. 3.0 Preparing the surface

Preparing the primer is vital to the success of the coating. It is therefore advisable to use Primus Naturale Spatula Stuhhi.

sez. 4.0 Application method

Segui il tuo Istinto is ready to use. To blend the colour, agitate the white base, then, once it has been mixed thoroughly, pour in the quantity of colouring agent specified in the catalogue and mix until the shade obtained is completely even. Before applying it, allow the coloured product to rest for at least 12 hours. Primus: Apply Primus Naturale with a roller. 1 coat : Apply in 1 coat the product by trowel, in abundant quantity. Pietra Spaccata: to obtain this effect, use our synthetic tool, which creates veins like marlbes. http://www.youtube.com/watch?v=x6DU6QlBdxI&feature=youtu.be Pietra Levigata: to obtain this effect, after the first coat, compact using the trowel. http://www.youtube.com/watch?v=hC6nJ14RPqM&feature=youtu.be Applicare la finitura desiderata (vedi sez. Finitura)

sez. 5 Finish

Cera del Vecchio: Mix the product for a couple of minutes before use. Leave the surface to be treated to dry for at least 48 hours. Apply the wax using a cloth, approximately ½ m2 at a time, and remove any excess with a soft cloth. After about 10 minutes, the surface can be polished by wiping over with a clean cloth (microfibre, cotton or linen). White Paint: apply with a sponge on Segui il tuo Istinto, which must have dried at least for 48 hours. Vetro: Protective water-based finish. On two coats of Spirito Libero, apply a first coat of Vetro diluted 90% with water, and a second coat undiluted.

sez. 6.0 Technical characteristics: application

| Dilution/Mixing | Ready to use |
| --- | --- |
| Consumption | 2 Kg/m2 in 1 coat |
| Tools for application | Steel spatula |
| Primer | Primus Naturale Spatula Stuhhi |
| Protective coating | Cera del Vecchio-Vetro |
| Application temperature | The ambient temperature must not be less than +5° C. Do not apply onto hot surfaces or those exposed to sunlight. |
| Drying time | Wall completely dry after 28 days |

sez. 7.0 Technical characteristics: the product

| Composition | 6-month matured slaked lime putty, marble powders, water |
| --- | --- |
| Specific weight | 1,56 +/- 0,05 Kg/l |
| pH after 30 days | 12,5 +/- 0,2 |
| Fire reaction | Class A1 (according to American and German tests) |
| Storage temperature | keep the pots sealed, in a suitable environment, at a temperature of between +5 and +30 °C. Keep away from extreme cold. |
| EU limit value for this product: 300 g/l (2007) / 200 g/l (2010) of VOC | 0 g/l colore bianco |
|  | 20 g/l colorato con ns. additivi coloranti |
| Colours available | As per our catalogue |
| Pot sizes | Kg – 24 - 8 |
| Warnings | Skin contact: immediately remove any contaminated clothing. Wash with plenty of soap and water all parts that have come into contact with the product. Contact with eyes: rinse immediately with plenty of running water, with your eyes open, for at least 10 minutes. If necessary, seek medical advice. Do not use eye drops or any type of ointment unless prescribed by your doctor. If swallowed: seek medical advice and bring the paint pot or the product technical or safety data sheet with you. |

sez. 8.0 Toxicological data

This product requires Xi labelling (irritating to the eyes and skin)