BUILD UP BASE

DESCRIPTION

BUILD UP BASE is a Cement based product especially formulated for the preparation of floors for Marmorino resin. It is a powder product, based on hydraulic binders, portland cement, special synthetic polymers, selected aggregates and special binders additives. It is easy to prepare and apply, with an excellent filling power and very limited shrinkage. Build up base is a low thickness compound(cement based) design as a base coat to be used in conjunction with fibre mesh for the preparation of floors before the application of  Marmorino resin product. Ideal also over existing tiles and suitable over underfloor heating(on top of the screed).

Note: Advanced Spirito Libero recommends that our entire range of lime based natural materials is used only by highly skilled or licensed tradespeople with extensive experience in this field. It is the responsibility of the applicator to understand the application method and the procedure involved in order to obtain a satisfactory outcome. Advanced Spirito Libero recommends that architects, builders and clients appoint only experienced applicators in the relevant lime-based materials to ensure they'll be able to deliver the same finish as per the sample shown.

We cannot guarantee that the existing substrate will not move causing our product to delaminate and crack. This is a substrate failure and the movement is beyond our control.   Concrete is variable by nature, cement source, age and quality as well as placement, curing and finishing methods. All this will have an impact on the final appearance/quality of the concrete substrate you will be applying over.  Advanced Spirito Libero will not be held responsible for these naturally occurring variations in the finished floor. This Includes subfloor or substrate cracking.

# colour range

BUILD UP BASE is produced in WHITE version.

# surface preparation

| TYPE OF SURFACE | PRELIMINARY TREATMENTS |
| --- | --- |
| Plasterworks | Coat of CONSOLIDATING PRIMER. |
| Old plasterworks | Brushing, eventual power washing and a coat of CONSOLIDATING PRIMER. |
| Concrete | Coat of CONSOLIDATING PRIMER, then one coat of PRIMUS NATURALE primer |
| Medium density | Coat of PRIMUS NATURALE primer. |
| Plasterboard | Coat of PRIMUS NATURALE primer. |
| Plaster | Coat of PRIMUS NATURALE primer. |
| Plastic coating | A coat of oil based primer then a coat of PRIMUS NATURALE primer. |
| Tiling | Washing with acid degreasing and sanding. |

BUILD UP BASE is sold in a white powder form and it has to be mixed with 25-35% of clean water (4.25-5.95 L per bag). Mix  until you obtain the full homogenization of the mixture.

APPLICATION:

BUILD UP BASE

Apply one coat by trowel over a prepared surface (previously prepared with one coat of "Consolidating Primer") and lay the fibre mesh making sure to smooth the surface even and flat.
When the surface is dried or as soon as you can walk on it apply another coat by trowel
But this time add more pressure to skim the product.
Let dry till the day after before starting apply Marmorino resin.

ENVRONMENTAL CONDITIONS FOR THE APPLICATION PHASE

The ambient temperature must be between +10°C and +30°C, while the relative humidity must be less than 85% for at least a day.

PRESERVATION

The product maintains its unaltered features for 6 months, if it is correctly preserved. Store it in a dry, close and covered places, away from humidity and rain.

SAFETY NORMS

Avoid contacts with skin and eyes, and in case wash immediately the part with water. For further information, consult our safety sheet.

DISPOSAL

This product should be disposed of in accordance with existing laws. Empty plastic vases or paper sacks must be recycled.

TECHNICAL DATA

|  | Fine white powder |
| --- | --- |
| Packaging | 17 Kg. net bag |
| Application tools | Trowel |
| Specific weight | 1,50 ± 0,05 Kg/l |
| Theoretical yield | 2/2.4 Kg/m2 in 2 coats |
| Grain size curve | 0,02 ÷ 0,70 mm |
| Mixing | With water 25 – 35% |
| Working life | 3 hours working life |
| Deep dry | 28 days |
| PH after 30 days | 12,5 ± 0,2 |
| Inflammability | Not inflammable - class A1 |